Quick Guide: EMG Feature Extraction & Classification Workflow

Overall Process Flow

- Main script (features_main.m) calls functions to process one subject at a time

- Manual subject ID change needed ('s1' → 's2', etc.) - run separately for all 15 subjects

- Two parallel pipelines: Time Domain (40 features) and Frequency Domain (6 features)

Step-by-Step Code Execution

1. Data Loading

- Loads subject .mat file (e.g., s1_delsysEMG.mat)

- Contains 3-channel EMG data from forearm muscles (2000 Hz sampling)

2. Preprocessing (in featuresextract_a.m)

- Signal normalization

- Bandpass filter: 45th-order FIR, 20-500 Hz (removes artifacts)

- Notch filter: 50 Hz IIR (removes power line interference)

3. Segmentation

- Window size: 300 ms (600 samples)

- Overlap: 50% (150 ms)

- Windowing options: Rectangular, Hamming, Hann, Blackman, Bartlett, Kaiser, Gaussian, Chebyshev

  - Manually uncomment desired window type before running

4. Feature Extraction (in getFeatures.m)

- For each channel (3 muscles):

  - Time Domain: 40 features (WL, ZC, RMS, VAR, etc.)

  - Frequency Domain: 6 features (Mean/Median Frequency, Band Power, etc.)

- Combines all channels → 120 features (40×3) for time domain or 18 features (6×3) for frequency domain

- Adds class labels (1-15 for finger movements)

5. Output

- Exports Excel file: features_extracted_subjectX.xlsx

- Contains features + labels ready for classification

Classification Phase (separate step)

- Load extracted Excel features into MATLAB Classification Learner App

- Test models: Decision Trees, SVM, KNN, Neural Networks

- Hyperparameters manually per model

Summary

- Total feature files will be 1680    (8 windowing x 7 classifiers x 2 sets x 15 subjects)

- 112 unique (window + classifier) combinations per feature set

- 1,680 total model training runs across all subjects

- Each combination needs manual hyperparameter tuning in Classification Learner App