Detail of Each Folder/File Uploaded in Github

Data / Experimental Protocol

The experimental protocol involved fifteen healthy volunteers performing fifteen distinct finger movements, which included individual, two-finger, and three-finger extensions. Surface EMG data was collected using three Trigno Avanti wireless sensors placed on the Flexor Carpi Radialis, Flexor Carpi Ulnaris, and Brachioradialis muscles of the left forearm. Each participant performed a 6-second movement followed by 4 seconds of rest, with this cycle repeated five times per movement. Recordings were conducted to capture a dynamic dataset, and the resulting EMG signals were sampled at 2000 Hz.

Description of Each File Uploaded at Github.

Following files /  folders are uploaded.

Folder: 6 Frequency Domain Features Extraction code files

It contains following three MATLAB m files with their descriptions.

- MATLAB Feature Extraction Script - features_main.m

This script extracts frequency-domain features from EMG data for a single subject at a time. It calls the featuresextract_a() function to process three EMG channels for a specified subject, combines the features, removes redundant columns, and creates a structured table with 18 features (6 frequency-domain features × 3 sensors) plus class labels. The user needs to manually change the subject identifier (e.g., 's1' to 's2', 's3', etc.) and run the script separately for each of the 15 subjects. The final feature table is exported as "features_extracted_subject1.xlsx" (or corresponding subject number) for classification.

Key features extracted: Mean Frequency, Median Frequency, Band Power, Occupied Bandwidth, Peak Amplitude, and Power Bandwidth from each of the three forearm muscles.

- MATLAB Feature Extraction Function - featuresextract_a.m

File Description for GitHub

This function processes raw EMG data for a specified subject and channel, applying signal preprocessing and feature extraction for upper limb movement classification.

Function Workflow:

- Input: Takes subject identifier (e.g., 's1', 's2') and channel number (1-3) as inputs

- Data Loading: Loads MATLAB data file containing 15 finger movement classes (individual, dual, and triple finger extensions)

- Preprocessing:

  - Normalizes raw EMG signals

  - Applies 45th-order FIR bandpass filter (20-500 Hz) to remove artifacts

  - Applies 50 Hz IIR notch filter to eliminate power line interference

- Segmentation: (Commented sections show options for 8 different time-domain windowing techniques - Rectangular, Hamming, Hann, Blackman, Bartlett, Kaiser, Gaussian, and Chebyshev windows with 300ms window size and 50% overlap)

- Feature Extraction: Calls getFeatures() function to extract features from each movement class

- Labeling: Assigns numeric labels (1-15) corresponding to each finger movement class

- Output: Returns combined feature matrix with class labels for all 15 movements

Note: The user must manually uncomment the desired windowing technique section before running. Different preprocessing windows can be selected to evaluate their impact on classification performance as described in the manuscript.

- MATLAB Feature Extraction Function - getFeatures.m

File Description for GitHub

This function extracts frequency-domain features from segmented EMG data using MATLAB's Signal Processing Toolbox.

Function Workflow:

- Input: Takes segmented EMG data matrix as input

- Processing: Iterates through six frequency-domain feature extractors

- Features Extracted:

  - Mean Frequency - Average frequency of the signal's power spectrum

  - Median Frequency - Frequency that divides the power spectrum into two equal halves

  - Band Power - Average power within the frequency band

  - Occupied Bandwidth - Frequency bandwidth containing 90% of signal power

  - Peak Amplitude - Maximum amplitude within frequency range (center selection)

  - Power Bandwidth - Frequency range where power is within 10 dB of peak

Output: Returns a feature matrix with 6 columns (one for each frequency-domain feature) for all segmented windows

Folder: 40 Time Domain Features Extraction Code files

It contains following three MATLAB m files with their descriptions.

- MATLAB Feature Extraction Script - features_main.m

Function Workflow:

- Calls featuresextract_a() function to process three EMG channels (Flexor Carpi Radialis, Flexor Carpi Ulnaris, and Brachioradialis) for subject 's1'

- Combines features from all three channels into a single matrix

- Removes redundant columns (positions 41 and 81) to clean the feature set

- Creates a structured table with 120 time-domain features (40 features × 3 sensors) plus class labels

- Features extracted per channel: Waveform Length (WL), Zero Crossing (ZC), Root Mean Square (RMS), Variance (VR), Standard Deviation (STD), Skewness (SKEW), Kurtosis (KURT), Slope Sign Change (SSC), Median (MED), Minimum (MIN), Maximum (MAX), Absolute Summation (ASM), Mean Absolute Deviation (MAD), Mean Energy (ME), Interquartile Range (IQ), Temporal Moments (TM3-TM5), V-Order (VO), Integral Absolute Value (IAV), Multiple Hamming Window (MHW), Mean Square Root (MSR), Absolute Sum Square Root (ASS), Integrated EMG (IEMG), Modified Mean Absolute Value (MMAV1, MMAV2), Difference Absolute Mean Value (DAMV), Difference Absolute Standard Deviation (DASDV), Difference Variance Value (DVARV), Approximate Entropy (ApEn), Maximum Fractal Length (MFL), Average Amplitude Change (AAC), Simple Square Integral (SSI), Log Coefficient of Variation (LCOV), Log Detector (LD), Coefficient of Variation (CV), Modified Mean Absolute Value (MMAV), Frequency Zero Crossing (FZC), Log Difference Absolute Mean Value (LDAMV), Cardinality (CARD)

Output: Exports "features_extracted_subject.xlsx" containing 120 features + labels for classification

Note: The user needs to manually change the subject identifier (e.g., 's1' to 's2', 's3', etc.) and run the script separately for each of the 15 subjects.

- MATLAB Feature Extraction Function - featuresextract_a.m

File Description for GitHub

This function processes raw EMG data for a specified subject and channel, applying signal preprocessing, segmentation (using Chebyshev window), and time-domain feature extraction for upper limb movement classification.

Function Workflow:

- Input: Takes subject identifier (e.g., 's1', 's2') and channel number (1-3) as inputs

- Data Loading: Loads MATLAB data file containing 15 finger movement classes:

  - Individual fingers: thumb, index, middle, ring, little

  - Two-finger combinations: thumb-index, index-middle, middle-ring, ring-little, little-thumb

  - Three-finger combinations: thumb-index-middle, index-middle-ring, middle-ring-little, thumb-ring-little, thumb-index-little

- Preprocessing:

  - Normalizes raw EMG signals

  - Applies 45th-order FIR bandpass filter (20-500 Hz) to remove artifacts

  - Applies 50 Hz IIR notch filter (Q-factor=45) to eliminate power line interference

- Segmentation: Currently configured for Chebyshev window (can be changed by uncommenting other window sections):

  - Window size: 300 ms (600 samples at 2000 Hz)

  - Overlap: 50% (150 ms)

  - Available windowing techniques: Rectangular, Hamming, Hann, Blackman, Bartlett, Kaiser, Gaussian, and Chebyshev

- Feature Extraction: Calls getFeatures() function to extract 40 time-domain features from each movement class

- Labeling: Assigns numeric labels (1-15) corresponding to each finger movement class

- Output: Returns combined feature matrix with 40 features + class labels for all 15 movements

Note: The user must manually uncomment the desired windowing technique section before running to evaluate different preprocessing approaches as described in the manuscript. Currently active: Chebyshev window with 60 dB ripple.

- MATLAB Feature Extraction Function - getFeatures.m

This function extracts 40 time-domain features from segmented EMG data for upper limb movement classification.

Function Workflow:

- Input: Takes segmented EMG data matrix as input

- Processing: Iterates through each segment window to extract comprehensive time-domain features

Features Extracted (40 total):

| # | Feature Name | Abbreviation | Description |
| --- | --- | --- | --- |
| 1 | Waveform Length | WL | Cumulative length of the waveform |
| 2 | Zero Crossing | ZC | Number of times signal crosses zero amplitude |
| 3 | Root Mean Square | RMS | Measures signal power |
| 4 | Variance | VAR | Amplitude dispersion around mean |
| 5 | Standard Deviation | STD | Amplitude dispersion around mean |
| 6 | Skewness | SKEW | Asymmetry of signal distribution |
| 7 | Kurtosis | KURT | Peakedness/flatness of distribution |
| 8 | Slope Sign Change | SSC | Counts slope sign changes |
| 9 | Median | MED | Central tendency |
| 10 | Minimum | MIN | Smallest recorded value |
| 11 | Maximum | MAX | Peak amplitude |
| 12 | Absolute Summation | ASM | Sum of exponential root values |
| 13 | Mean Absolute Deviation | MAD | Average absolute signal deviation |
| 14 | Mean Energy | ME | Average signal power over time |
| 15 | Interquartile Range | IQ | Signal variability measure |
| 16-18 | Temporal Moments 3-5 | TM3-TM5 | Skewness, kurtosis, and distribution magnitude |
| 19 | V-Order | VO | Higher-order statistics |
| 20 | Integral Absolute Value | IAV | Summation of absolute values |
| 21 | Multiple Hamming Window | MHW | Hamming window analysis |
| 22 | Mean Square Root | MSR | Signal energy and variability |
| 23 | Absolute Square Root Sum | ASS | Signal energy measure |
| 24 | Integrated EMG | IEMG | Signal area under curve |
| 25 | Modified MAV Type 1 | MMAV1 | Emphasizes signal subsets |
| 26 | Difference Absolute Mean | DAMV | Consecutive EMG differences |
| 27 | Difference Absolute STD | DASDV | EMG variability quantification |
| 28 | Difference Variance | DVARV | Variability in consecutive samples |
| 29 | Approximate Entropy | AENT | Signal complexity measure |
| 30 | Maximum Fractal Length | MFL | Complexity and variability |
| 31 | Modified MAV Type 2 | MMAV2 | Refined EMG analysis |
| 32 | Average Amplitude Change | AAC | Consecutive EMG differences |
| 33 | Simple Square Integral | SSI | Total signal power |
| 34 | Log Coefficient of Variation | LCOV | Relative variability |
| 35 | Log Detector | LD | Logarithmic energy |
| 36 | Coefficient of Variation | CV | Dispersion around mean |
| 37 | Mean Absolute Value | MMAV | Weighted MAV |
| 38 | New Zero Crossing | FZC | Signal frequency and muscle activity |
| 39 | Log Difference Absolute Mean | LDAMV | Signal changes between points |
| 40 | Cardinality | CARD | Unique values quantifying diversity |

Output: Returns a feature matrix with 40 columns (one for each time-domain feature) for all segmented windows

Note: This function implements the complete 40 time-domain feature set described in Table 2 of the manuscript, providing comprehensive signal characterization for accurate movement classification.

Folder: Subjects IEEE Data Port Link.docx

This folder contains the link i.e. https://ieee-dataport.org/documents/upper-limb-emg, from which you can download raw surface EMG data of fifteen subjects (s1_delsysEMG.mat to s15_delsysEMG.mat). Each .mat file holds a 3-row matrix representing signals from three forearm muscles, recorded at 2000 Hz while subjects performed fifteen distinct finger movements.

File: Classifier Learner App Hyper Paramters.docx

This folder contains different classification models trained using MATLAB's Classification Learner App, including Decision Trees, SVM, KNN, and Neural Networks. Each model's optimized hyperparameters are documented after feature extraction from Excel, providing a comprehensive benchmark for comparative analysis.

File: Description.docx

This document provides a step-by-step guide for extracting time and frequency domain features from EMG data across 15 subjects, using 8 different windowing techniques during preprocessing. It then explains how these feature sets are fed into 7 classifiers in MATLAB's Classification Learner App, resulting in 1,680 total model training combinations that require manual hyperparameter tuning.

File: Citation.docx

Citation reference file for the published PLOS ONE article, formatted for easy use by researchers referencing our work.